CLRC Instrumentation Department

Problem Report


Problem Report 



 Report Number :- 14

Project Name:-CMS-FED Throughout 

Item:-




PC 3025M Mk1

Problem:

It is noted that many capacitors have been tracked and viaed rather than the via being very close to the pad. 

The pad and via need to be as close as possible to reduce the series inductance.



Originator (sign, date)

Project Manager(sign, date)

Remedial Action:

The tracking of decoupling caps should be examined, for example C85 against C47. The tracking length should be minimal.

The tracking of some decoupling capacitors has been improved for CALICE

Project Manager (sign, date)



Last modified 24/06/03 14:27

page 1 of 1

