CLRC Instrumentation Department

Problem Report


Problem Report 



 Report Number :- 31

Project Name:-CMS-FED Back  end

Item:-




PC 3025M Mk1

Problem:

Voltage on QDR SRAM JTAG port exceeds datasheet specification. 

I cannot see any level translators or open collector outputs with pull ups to 2v5 to drive the JTAG inputs to the QDRs (u99, u100).

The relevant spec is given below.


Micron data sheet[image: image1.jpg]z 512K x 18
,fﬂcron‘ V Voo, HSTL, QDRb2 SRAM
TAP AC TEST CONDITIONS Figure 4
[rpr— Vo2s Tap AC Output Load Equivalent
nput ise and fall imes ns a5y
opus timing eference evel 125y
Output reference levels 125 00
Test oad termination supply voltage ... 125V
0—f)
7500 2098
TAP DC ELECTRICAL CHARACTERISTICS AND OPERATING CONDITIONS
(420°CS Ty £ +110°C, 249 = Vao < +26V unles otherwise noted)
DESCRIPTION Cowpimions _[svmpoL] _wiN_| max | uns | wores
It High (Logic ) Vortage Vi 7 [er03] v ] 12
it Low (Logic 8 Vortage e o [ o [ v 7
i Leakage Corrent ETET ™ L T Y
Dutput Leakage Carrent Curpus) dsabied, o T T Y
0V Vit Voo
it ow Valtage Var v
Gt Low Voltage Voz v
Gt igh Voltsge Vo T v
Gutput High Vortage Vs |17 v

NOTE: 1. Allvoltages referenced to Vs (GND)
2] Overshoot: | Vi (A0 < Voo + D7V for < KHKHEZ
Undershoot: Vi (AG) = 05V for ¢ < KHKH2
Fo

upe Vi< +2.6V and Voo 2.0 and Voo < 1.0V for £ 200ms

Ouring normal operation, Vec must not exceed Vo, Control input sl (such 3104, RANE, ) may nat have.
pulse vadihs s than 'HKL (MIN) r operate a fequencies exczeding ‘KF (MAX)






Originator (sign, date)

Project Manager(sign, date)

Remedial Action:

Revise schmatics before next production.

Project Manager (sign, date)



Last modified 24/06/03 10:58

page 1 of 1

