CLRC Instrumentation Department

Problem Report


Problem Report 



 Report Number :- 32

Project Name:-CMS-FED back end

Item:-




PC 3025M Mk1

Problem:

Contention problem GRANT_SWAP# 

R6 and RN36 can contend, circuit revision required.

The following need to be renamed

VME_GRANT_SWAP# to SWAP_GRANTED

GRANT_SWAP# to GRANT_SWAP

SWAP_REQ# to SWAP_REQ on U123 and PL19

SWAP_RQ# to SWAP-RQ (U123 Pn 10-A) & (U107 pin N6)

Connect R339 to GND instead of 5VSUP

Connect PL20 to 5VSUP instead of GND 



Originator (sign, date)

Project Manager(sign, date)

Remedial Action:

Revise schmatics

Project Manager (sign, date)



Last modified 24/06/03 15:16

page 1 of 1

