CLRC Instrumentation Department

Problem Report


Problem Report 



 Report Number :- 35

Project Name:-CMS-FED back end

Item:-




PC 3025M Mk1

Problem: Various minor improvements, 

To improve the reset performance the following changes should be made 

Discharge resistors fitted to power planes, 1.5V 1Kohm, 2.5v 1kOhm, 3.3V 1Kohm 5V 1Kohm and 12V 5.6k 

A 47CT0205 diode to be fitted in parallel with C789, anode to 5Vneg, this is to reduce the swing on 5V neg when the 12V to -5 switcher is off. 

There are several LED errors 

TSHDOWN led sense needs reversing, remove existing connection between R429 and U129 pin8 Connect R429 and pin 12. 

Calice would like Leds to come on in the Active condition; 



Originator (sign, date)

Project Manager(sign, date)

Remedial Action:

TSHDOWN led sense needs reversing, remove existing connection between R429 and U129 pin8 Connect R429 and pin 12. 

Calice would like Leds to come on in the Active condition; 

R1221 R1222, R1223, R1225, R1226, R1227, R1228 dis connect Pin 2 from GND and connect to 5VSUP 

R1224 connect disconnect pin2 from GND and connect to 5VSUP, 

Rotate DS81 to D88



Project Manager (sign, date)



Last modified 15/07/03 16:38

page 1 of 1

