CLRC Instrumentation Department

Problem Report


Problem Report 



 Report Number :- 43

Project Name:-CMS-FED 

Item:-




PC3205M Issue 1

Problem:
The PCB is being made by another Board Manufacturer and they can’t achieve the same impedances with the existing board build. To achieve the closest impedance figures to the original design the track widths will have to change, as well as the specified impedance figures.

Originator (sign, date)     Chris Day     18/12/03

Project Manager(sign, date)    John Coughlan     18/12/03

Remedial Action:

Existing Board -  Issue 1

1. Obtained new build from Ddi (Board Manufacturer)

2. Adjusted the drilling drawing, modified the impedance tables to the new impedance figures and new track widths. Drilling Drawing now issue B

NOTE: PCB still remains at issue 1, Ddi will adjust the tracks widths in their front end department, to achieve the required impedances.

For Issue 2 Board

1. Track widths will be changed to Ddi’s build on the pcb design at RAL.

Project Manager (sign, date)



Last modified 18/12/03 15:14

page 1 of 1

