CLRC Technology Department

Procedure 9010 Form 1.0

Route Card
Project

CMS FED
Product

Front End Driver
Identification Ser. No. PC3205M/1      012



manufacturing process / supplier
Batch ID / date

PCB manufactured by DDi
6 off January 2004

Assembled by DDi
6 off March 2004




operation
Comments
initial
date

Visual Inspection
Passed
ITC
11/03/04

Modifications according to PR003


Done
ITC
11/03/04

Modifications according to PR004


Done during assembly
ITC
11/03/04

Modifications according to PR005


Done during assembly
ITC
11/03/04

Modifications according to PR006


Done during assembly
ITC
11/03/04

Modifications according to PR009


Done during assembly
ITC
11/03/04

Modifications according to PR015


Done during assembly
ITC
11/03/04

Add a 100R resistor in parallel to the inputs of the 96 ADC channels.
Done during assembly
ITC
11/03/04

Current trip
Change R396 to 33K to increase the 3.3V trip to 20A
ITC
11/03/04

JTAG tests
Failed. Dry joint on pin 10 of U129 and R361.
ITC/ RMM
12/03/04

Fibre optic ramp tests
Failed. Dry joints on 100R ADC resistors, dry joint on U21_6.
ITC
15/03/04






















































































PAGE  
1

