[image: image1.jpg]&' THG 8430

ESD DISCHARGE STRIP
B et

=i
e

L
4]

DO®

CMS TRACKER FED v2

RUTHERFORD APPLETON (
H PC3205M/2 NE
o Y (






CMS-FED PC3205M Test Specification for DDi Technologies

Document History

V1.0
22.06.04

JAC
Skeleton draft.

V2.0
10.11.04

ITC
Added test procedures up to and including Boundary Scan.

V2.1
11.11.04

JAC
added comments.

V2.2
22.11.04

ITC
Continued expansion and re-ordering of testing flow chart.

Introduction

The CMS-FED is a 9U x 400 mm VME64x board (Figures 1 & 2) used to readout data from the CMS experiment at CERN Geneva. A total of 500 boards are required for the complete readout system.

This document describes the testing station and commissioning procedures to be performed at DDi Technologies in order to verify the manufacture of CMS FED PC3205M prior to delivery to RAL.

These tests are to be performed after the standard Quality Assurance methods such as Automated Optical Inspection and X-Ray/Ersascope inspection described elsewhere [1].

It is assumed that the procedures described will be carried out by technicians employed at DDi following basic training by RAL staff and that thereafter only minimal supervision and support should be necessary.

The connectivity of the digital components will be verified by means of standard JTAG Boundary Scan. The connectivity of the analogue circuitry and the verification of the board operation will be carried out by standalone VME crate based tests using a test station provided to DDi. The latter tests will be carried out using VME readout but without the use of optical input signals.

The results of the Boundary Scan, and Analogue tests and the visual checks of the operators will be recorded in the FED Route Card Online Database along with details of any repair work carried out.

The final system tests, using optical inputs and fully populated crates, will subsequently be carried out at RAL. Boards will then be shipped to CERN for final CMS readout integration tests at Prevessin B904 before installation at USC55. These tests and the associated evaluation of the hardware and firmware design are beyond the scope of the present document.

[image: image16.wmf]Phase 1

Pre Boundary Scan test

set-up

Phase 2

Boundary Scan

tests

Phase 3

Post Boundary

Scan test set-up

Phase 4

Digital circuits

tests

Phase 5

Analogue circuits

tests


Figure 1 : CMS-FED PC3205M/2 primary side (N.B. optical fibre and front panel connector shown will not be present at DDi)

[image: image2.jpg]e
GaTonos0m
@029202°%

% Goassezen
s mEhEs o

3
% ___ATTENTION
i
a La
B
£
aslie
T B
3

o

co0c0e

PC3205M/2





Figure 2 : CMS-FED PC3205M/2 secondary side

Test Set-ups

The proposed set-ups are shown below:

[image: image15.wmf]Phase 1

Pre Boundary Scan test

set-up

Phase 2

Boundary Scan

tests

Phase 3

Post Boundary

Scan test set-up

Phase 4

Digital circuits

tests

Phase 5

Analogue circuits

tests







Figure 3 : Boundary Scan test set-up

[image: image3.jpg]A NGRS AN A N AN NN S NS S

ARA AR

R e e e et —

canTe 1

o mi——





Figure 4 : Image of existing RAL Boundary Scan set-up






Figure 5 : VME readout test set-up

The equipment to be provided by RAL to DDi are listed below:

Item
Comment

LHC standard 9U crates.
Two off, one for the Boundary Scan and one for VME testing.

PC running Linux.
CERN Red Hat Linux

PC-VME interface cards.
SBS or CAEN

Loop back card and cables for Boundary Scan 
Loop back card is used to test as many as possible of the J1 and J2 connections on the FED and is plugged into an adjacent slot on the front of the backplane.

PC/laptop with Boundary Scan system installed on it, and USB splitter box to generate the four boundary scan chains.
JTAG Systems to match RAL system. May be available at DDi?

Boundary Scan files.


Bar codes & 2D barcode reader.
DENSO QS20P-A hand scanner

Compact flash cards & Files





N.B. Identical set-ups will also be required at RAL and CERN.

Test Procedures

The results at each step of test procedures described in the following sections (Figure 6) are entered into the LabView program (section N) for storage in the online Route Card Database.

Figure 6 : Test procedures Phases

Phase 1: Pre Boundary Scan test set-up
1. Visual inspection

2. Fit 1.5A fuse

3. Check for short circuits on the supplies with a multimeter.

4. Fit serial number to PCB

5. Fit front panel. 

6. Fit front panel serial number label.

7. Attach 2D barcode to front panel and record details (image 1.1).

8. Fit shorting links for testing (image 1.2, 1.3  & 1.4).

9. Fit diagonal air deflector bar (image 1.5).

10. Insert carefully into Boundary Scan test crate and check that the card inserts and extracts correctly.

11. Power on the crate and verify that the card powers successfully by checking the LED status (image 1 .6), all LEDs off.

12. Enter the details of all these tests into LabVIEW software.

[image: image4.jpg]-v

RAL. 2065 |
SERIAL NO. |




  [image: image5.jpg]= Tpag ™ =

-

2 LEOOUSH
JWR0ZSU @
L9EL0vAaH

ea el
‘e R121T
0 w ed







image 1.1




image 1.2

[image: image6.jpg](C611 ::o‘
.a~2+

nmquuuw A iamuo

wu.;




  [image: image7.jpg]CERN 7/2003
MODIFIED

TTCRX 32

By LJ.,_ -

fl,
m— C551 1 C525
—— .

G e e ——

¥ o w
| = jhi
- o
50 L0 L OLD.
Se=gmslas
N O

.
T

= Teer 1 “rp100
© TPas ™= =g,
-ﬂ..:.: o \JE- o
N..icu ik 7T
TPO4 == gy
S a1 = g ”

£ janizes
N:E. = =rpez *

A
1

[ﬁ:n

rjfﬁ'f_“

.
et
mCrss Dy

Sen cTo4

T : - _ ¥— ¢ [rr—






image 1.3




image 1.4

[image: image8.jpg]‘ oo
b1 e * ~

; o)

‘. - v &
n3sz ‘ ~SA
e (L



  [image: image9.jpg]o e
&N e =

R122413 s g z :

v R1223-Tikmmmas ™ =S ==
16 mzzdm*_‘ggr, 5-’?.': = 3 N i = C763
238 m22zummewd STE oL - :
Ri227-trkmmsin sy E@ s L |
R1228-1 ikt =% S Z o w@> \ R26-11 = o =

Ds8s j;-,,_«j'?isl—,jé;ﬁ L e

crs3 U129

€779 c187

oy — —— et







image 1.5




image 1.6

image 1.7

Phase2: Boundary Scan tests

1. Select ident test and run. This will check the identity of the devices in the JTAG chains. Note 1
2. Select inter test and run. This will check the interconnection of the devices in the JTAG chains. Note 1
3. Enter results file into LabVIEW software so that it copies and renames the results file to include the serial number.

Note 1.  More details of the interpretation of the software results are discussed later.

Phase3: Post Boundary Scan test set-up

1. Plug-in programmed compact flash card.

2. Insert into system test crate and power on.

3. Connect Xilinx “parallel 3” cable to the VME Boundary Scan connector and program the VME PROM device.

4. Re-boot PC3205 card by pressing the restart button and check to see that DS1_X, DS2_X, DS5_X, and DS7_X are all flashing (image 1.8).  During the loading phase DS80 (amber LED on the back of PCB under U111) should flash and then be on constantly when it has finished, if DS14 (red LED next to DS80) is on the an error in reading the Compact Flash card has occurred.

[image: image10.jpg]S AT ]
l’ s
l\'

-l (- o e
- s a® -s
- -

™ & & i
g B

0 0
COMe e el 2 o g £ 8
o o

Eobot bt A
SN © (f oo

-~

o

=]

G5 50 O g

bioloo oo
[ oo %"nlb & oo
N

and D88

(Sould be OFF)

B O 0D 0O
S HOOD . 0D o000
obo voowok
e 0. 6¥88 - &ﬂoﬂo
.U (o] Oaﬂ. .

0. O

4

=
g i
DS6_X

DS5

J

(Flashing

DS4 X

DS2_X

o

DS3_X

v

DS1_X





image 1.8

Phase 4: Digital circuits tests

1. Carry out the digital test section of the LabVIEW software by clicking on the GO button, this section will test the operation of the temperature sensors, voltage monitors, and check to see that the correct versions of code have been loaded into the FPGAs via the compact flash card. Some screen shots can be seen below.

[image: image11.jpg]italTestExecutor. S [=T S|
Fie_Edt_Operte Tools WidowHln o
7

Mein | FPGA Fimvare Chek | 7 |

) ) Feardng Data From FED.
———

Pass

Fail

FED Serial Number:| 004

Digital Test Number: 1




  [image: image12.jpg]HE o =lolx|

Fle_Ect_Operte Tools Wincow_Help o

7
| Fpa Frmer crck | 7 |
FE FPGA 0 FEFPGA 1
e Vs e Vs
00017 Za0onat7
g stats oG tats
E FPGA0: O FE FPGA1: O
SHE R FE FPGA 2 FE FPGA 3
i UL Firmware Version: Firmyare Version:
EITETE] 23000317 23000317
it FPGA Status: FPGA Status:
et st RS R
BE FPGA FE FPGA 4 FEFPGAS
e s e Vs Fimuae Versin:
Zznonass 00t 25000317
g sats g sats PG St
GE PG Acorssd FPGA, E FPGAG: O & FPGAS: 0K
e s et
FE FPGA S FE FPGA 7
e Vs e Vs
Zo0nat7 ey
FPGA Status: FPGA Status: B
FE FPGAG: OF FE FPGAT: Ok





Phase 5: Analogue circuits tests

2. Carry out the analogue part to the LabVIEW test software, by controlling the TrimDACs this checks the operation of the trimDAC itself, the analogue circuitry (not the OptoRX device), and the functionality of the AD9218 ADC chips. Some screen shots can be seen below.

[image: image13.jpg]Fle Edit_Operate Tools Window Help

Main | Voltages & Temperatures  THmDAC & Opto Fix Access Test ‘ THmDAC & Opto Fix Channel Test

J )
re unito] || Fe unit 1] || FE Unit 2 || FE unit3 ] || FE unita] || FE Units] || FE units ||| FE Unit 7]

Currently Viewing FE Uni 0

FE Unit Channel 0 FE Unit Channel 3 FE Unit Channel 6 FE Unit Channel 9
e I e e e
Trim DAC Access Test OK 2| Trim DAC Access Test OK 4| i DAC Access Test OK 4| Trim DAC Access Test OK 2|

FE Unit Channel 1 FE Unit Channel FE Unit Channel 7 FE Unit Channel 10
Trim DAC Access Test OK 4| T DAC Access Test OK 4| Trim DAC Access Test OK =] i DAC Access Test OK 4|

FE Unit Channel 2 FE Unit Channel 5 FE Unit Channel 8 FE Unit Channel 11
o | G| | o | TR | e iR ]| |

Tim DAC Access TestOK  +| | | |TimDAC AccessTest OK | | | |TrimDAC AccessTest OK | | | |TrmDACAccessTestOK o
Opto R Access Test OK Opto RX Access Test OK Opto R Access Test OK Opto R Access Test OK




  [image: image14.jpg]~=lolx|

Main | Voltages & Temperatures

THMDAC & Dpto Fs Access Test

THMDAC & Opto s Channel Test

FE Unit 0] FE Unit 1] FE Unit 2| FE Unit 3| FE Unit 4] FE Unit 5| FE Unit 6] FE Unit 7]
info info info wol | ool o] | [cnad [evel | [-cnad [ info
info! info! info! info! u info; u info; u info; info.
o o o wol | (zaal [imel | {cual Lol | [zl [ o
o o o wol | gy ool | [ onad [me] | | ena e o
e e e wo] | Wl o] | (cudl [mo] | el [me o
o o o o] | ons [e] | ond o] | |onsa [e o
o o o wo] | [cned [re] | [-chad [mo] | |-cnad [me o
o o o wo] | on o] | ocnl [me] | {cnzi [me o
e e e o] | Jcns [me] | (ocul [me] | fcnad [ime o
e e e wo] | Jenall o] | (el [mo] | ficnsd e o
o o o o] | et [o] | sl [mo] | el [e o

ADC Counts

0 1o
TmDAR Oifeet

20

210 255

Currently Viewing|

FEUnt:  FE Charel
7 i

TrmDAQ Data) Q Opto Fix Data)

Charnel Status
TMDAC Test Resut: ADE vales are not
monotoric

Opto R Input Offset Test Resul: 0K
Opto s Dutput Dffset Test Recult: OK





Test Software

The test software is a LabView based program running under Linux.

The operator interface GUI is illustrated in Figure N.

References

[1] DDi Production Report for PC3205M/1, March 2004

Crate with loopback card for Boundary Scan testing.





PC/Laptop with JTAG software





JTAG USB interface box





FED





VME I/F





VME Crate 1





VME Crate 2





Linux PC





� EMBED Visio.Drawing.6  ���








6

_1162621872.vsd
�

Phase 1
Pre Boundary Scan test set-up�

Phase 2
Boundary Scan tests�

Phase 3
Post Boundary Scan test set-up�

Phase 4
Digital circuits tests�

Phase 5
Analogue circuits tests�


